Packing List Include:

1 ×Load Cell Amplifier

1 ×User Manual

ATO Load Cell Amplifier with 6-digit LED display, output signal 4-20mA/ 0-10V/ RS485(Modbus-RTU)/ 4-20mA & RS485/ 0-10V & RS485. It can be used as a load cell indicator for reading the weight values.

There are 3 keys for parameter settings. Digital load cell amplifier can be calibrated on-line by its 3 keys. The setting parameters include zero, capacity, communication rate, division value, decimal digits, etc.

How to restore factory settings

When setting error, you can press K2 key to restore factory setting, free from maloperation.

Step 1: Power off → long press K2

Step 2: Power on → wait 2 seconds, then release K2

ATO Load Cell Amplifier Specification

| Model | ATO-S-LCTR-DY610 |
| --- | --- |
| Accuracy | ±0.2%FS ±1 byte |
| Input signal | 0~±20mV |
| Excitation voltage | DC 5V (for load cell) |
| Output signal * (optional) | DC 4-20mA,DC 0-10V, RS485 (Modbus-RTU), 4-20mA & RS485, 0-10V & RS485 |
| Sampling rate | 60 times/s, 120 times/s |
| Power supply | DC 12-30V |
| Power consumption | ＜1W |
| Display | 6 digit red LED, -99999~999999 |
| Readout units | kg, g, ton, and newtons |
| Protection class | IP65 |

ATO Load cell Amplifier wiring diagram :

The shield of the load cell can be connected to the amplifier's E- terminal together.